### PART 1 - GENERAL

- DESCRIPTION

  - Work included:

    - Bronze double weatherstripped swing door configurations.

    - All door anchors, mullions, covers and trim.

  - Related work specified elsewhere:

    - Glass, glazing and glazing materials, Section 08810.

    - Perimeter caulking, Section 07915.

    - Miscellaneous structural items, Section 05100.

- QUALITY ASSURANCE

  - Manufacturer shall have not less than 10 years experience in the fabrication of heavy custom steel doors and be a member of The Steel Window Institute (SWI).

  - Installation of doors shall be done by experienced door installers.

  - Allowable tolerances: Size dimensions + 1/16 inch.

  - Source quality control:

    - All doors shall comply with the International Building Code (IBC), the International Residential Code (IRC), the Florida Building Code (FBC) and the Dade County Product Control Division.

    - Air infiltration test

      - Products must be independently lab tested and meet or exceed ASTM E283 or Miami-Dade Testing Application Standard TAS 202.

      - Maximum air infiltration 0.30 CFM/SQ Ft. with differential pressure across window unit of 1.57/ 6.24 PSF.

    - Water penetration test

      - Products must be independently lab tested and meet or exceed ASTM E331 or Miami-Dade TAS 202.

      - No water penetration for 15 minutes when window is subjected to a rate of flow of 5 gal./hr./sq. ft. with differential pressure across window unit of 4.50 PSF (or 15% design pressure for areas requiring HVHZ).

    - Field test

      - Field testing criteria (when applicable) shall be in accordance with AAMA 502-12

    - Structural test

      - Meets or exceeds ASTM E330 or Miami-Dade TAS 202.

    - Forced entry test

      - Meets or exceeds ASTM F588 or Miami-Dade TAS 202.

      - Grade 40 @ 300 pounds

    - Impact test – Large missile

      - Meets or exceeds FBC ASTM E1996 or Miami-Dade TAS 201.

      - Withstand 2x4 missile impact at all designated contact points without penetration.

    - Cyclic wind loading test

      - Meets or exceeds FBC ASTM E1886 or Miami-Dade TAS 203.

      - No failure or distress after subjected to the cyclic sequence to a design load (positive and negative.)

    - Upon request, the door manufacturer shall provide a test report from a qualified independent U.S. testing laboratory regularly engaged in testing doors to verify that products conform to test requirement as outlined.

- PRODUCT, STORAGE AND HANDLING

  - The General Contractor shall be responsible for the protection and storage of the doors after delivery to the site.

  - Store in designated areas in an upright position on wood slats or on a dry floor in a manner that will prevent damage.  Ventilate canvas or plastic coverings to prevent humidity buildup.

- WARRANTY

  - Provide Hope’s standard 10 year Limited Material Warranty.

  - This guarantee is not intended to cover the bronze finish or any protective coating.

PART 2 - PRODUCT AND FABRICATION

- Manufacturers:

  - Furnish all labor and materials to complete the fabrication of windows as shown on architect's drawings and as specified herein.  All windows covered by this specification shall be domestically manufactured in the U.S.A.

  - Manufacturers:  Subject to compliance with requirements covered in this specification, provide products by Hope’s Windows, Inc. – Jamestown, NY (phone: 716-665-5124, e-mail: sales@hopeswindows.com) whose name and products are used to establish the standard of workmanship.

  - Substitutions:  Written approval necessary 10 days prior to bid through submission of the following:

    - Full-size window/door samples matching required scope.

    - Applicable test reports as outlined in Quality Assurance Section 1.2

    - List of (5) recently completed projects of similar size and scope.

- MATERIALS

  - Double weatherstripped doors shall be manufactured from solid extruded bronze profiles.

    - Profiles shall be extruded bronze Alloy 385 with a minimum thickness of .141 inches.

    - Perimeter frames and ventilator profiles shall have glazing rebates providing an unobstructed glazing surface of at least 3/4".

    - Glazing rebate surfaces must be perpendicular to the web or stem of the section.  Applied glazing rebate extensions and rebate surfaces that are tapered will not be acceptable.

    - Combined weight of frame and ventilator profiles shall be a minimum of 4.40 pounds per lineal foot. Frame section alone shall not weigh less than 1.95 pounds per lineal foot.

    - All bronze profiles must be a minimum of 1-3/4″ in depth.

    - The ventilator profiles shall have integral grooves located in the exterior and interior bedding contacts for the reception of double weatherstripping.

    - Kick panels shall be made from two, 1/8 thick bronze sheet with a rigid polyisocyanurate insulation core.

  - Muntins shall be manufactured from solid extruded bronze profiles:

    - Profiles shall be extruded bronze Alloy 385 with a minimum thickness of .141 inches.

    - Glazing rebate surfaces must be perpendicular to the stem of this section.  Rebate surfaces that are tapered will not be acceptable.

    - The 1-3/8” tee shall weigh 1.664 pounds per lineal foot and the 7/8” tee shall weigh 1.375 pounds per lineal foot. (Specify).

  - Lockbox and kick panels shall be manufactured from solid extruded bronze profiles or bronze sheet.

  - Glazing beads shall be extruded bronze.

  - Weatherstripping shall be extruded vinyl, EPDM closed cell sponge, flexible silicone or polyethylene clad urethane foam.

  - All screws that are furnished by Hope’s, for hardware, trim, covers, anchoring, weatherbars, water dams, screens, etc. shall be non-ferrous brass or stainless steel.

- FABRICATION

  - Fabricate bronze doors in accordance with approved shop drawings.

  - Corners of frame and ventilator shall be mitered and back welded.

  - Muntins shall be coped and welded to the perimeter frame.  Muntin intersections shall be slotted, cross notched and welded.

  - Lockbox and kick panels shall be cut square and coped to suit then screw attached to the door.

  - Prior to fabrication the 1/8 thick bronze skin shall be laminated to rigid polyisocyanurate insulation core. Kick panels shall be factory glazed with a structural silicone sealant.

  - Glazing:

    - All doors shall be designed for inside or outside glazing.

    - Provide replaceable continuous glazing beads to suit the glass specified.

    - Glazing beads shall be cut and shop fitted to each glass lite prior to shipment.

    - Manufacturer to provide correct glazing wedge and tape in accordance with the tested assembly.

    - All glazing pockets shall be back sealed for weather tightness by field glazier.

  - Weatherstrip.

    - Continuous weatherstripping shall be applied to the integral weatherstrip grooves in the interior and exterior contact surfaces of the frame and ventilator profiles. Weatherstripping that is surface applied or requires an additional retainer or requires screws for application shall not be acceptable.

  - Bronze Insect Screens (Optional).

    - Bronze screen frames shall be finished to match the doors.

    - Bronze screens shall be rewireable to allow for mesh replacement.

    - Bronze screen fastenings shall permit easy attachment and removal from the interior.

  - Finish (select from the following):

    - Factory Finished (specify finish)

    - Field finished by others

- OPERABLE HARDWARE

  - Pivots:

    - Aluminum-bronze pivot with stainless steel pin

  - Thresholds

    - Threshold shall be as shown (select material from items a or b):

      - Aluminum (anodized)

      - Bronze (toned)

  - Single Leaf (exterior and interior operation):

    - Bronze lock box fitted with multipoint locking device with exterior and interior lever handles with escutcheon or rosette trim

    - Interior thumbturn

    - Exterior key cylinder

  - Double Leaf (exterior and interior operation):

    - Active leaf:

      - Bronze lock box fitted with multipoint locking device with exterior and interior lever handles with escutcheon or rosette trim

      - Interior thumbturn

      - Exterior key cylinder

    - Inactive leaf:

      - Exposed flushbolts

      - Dummy trim with exterior and interior non-operable lever handles with escutcheon or rosette trim (optional)

  - Mortise lock and cremone hardware are also available; consult Hope’s.

PART 3 - EXECUTION

- INSPECTION

  - Door openings shall conform to details, dimensions and tolerances shown on the door manufacturer's approved shop drawings.

  - Conditions which may adversely affect the door installation must be corrected before installation commences.

  - The wash down of the adjacent masonry or surrounding substrate must be completed before erection commences to prevent damage to the finish by the cleaning materials.

- INSTALLATION

  - Doors specified under this section shall be installed by experienced personnel.

  - Install doors in openings in strict accordance with approved shop drawings.

    - Set units plumb, level and true to line, without warp or rack of frames.

    - Anchor units securely to surrounding construction with approved fasteners.

    - The exterior joints between the doors, trim and mullions shall be properly sealed water-tight with an approved sealant and neatly pointed.

  - Attach door hardware, as required, and adjust doors to operate smoothly free from twist and to be weather-tight when closed.

  - Repair any abraded areas of the factory finish.

- CLEANING

  - Door installer shall leave door surfaces clean after installation and ready to receive glass and glazing.  The door installer will not be responsible for final cleaning.