### PART 1 – GENERAL

- DESCRIPTION

  - Work included:

    - Bronze double weatherstripped windows with fixed, project-in, project-out, side hung-in or side hung-out configurations.

    - All window anchors, mullions, covers and trim.

    - Stainless steel insect screens for all operating ventilators. (optional)

  - Related work specified elsewhere:

    - Glass, glazing and glazing materials, Section 08810.

    - Perimeter caulking, Section 07915.

    - Miscellaneous structural items, Section 05100.

- QUALITY ASSURANCE

  - Manufacturer shall have not less than 10 years experience in the fabrication of heavy custom steel doors and be a member of The Steel Window Institute (SWI).

  - Installation of doors shall be done by experienced door installers.

  - Allowable tolerances: Size dimensions + 1/16 inch.

  - Source quality control:

    - All windows shall comply with the International Building Code (IBC), the International Residential Code (IRC), the Florida Building Code (FBC) and the Dade County Product Control Division.

    - Air infiltration test

      - Products must be independently lab tested and meet or exceed ASTM E283 or Miami-Dade Testing Application Standard TAS 202.

      - Maximum air infiltration 0.30 CFM/SQ Ft. with differential pressure across window unit of 1.57/ 6.24 PSF.

    - Water penetration test

      - Products must be independently lab tested and meet or exceed ASTM E331 or Miami-Dade TAS 202.

      - No water penetration for 15 minutes when window is subjected to a rate of flow of 5 gal./hr./sq. ft. with differential pressure across window unit of 4.50 PSF (or 15% design pressure for areas requiring HVHZ).

    - Field test

      - Field testing criteria (when applicable) shall be in accordance with AAMA 502-12

    - Structural test

      - Meets or exceeds ASTM E330 or Miami-Dade TAS 202.

    - Forced entry test

      - Meets or exceeds ASTM F588 or Miami-Dade TAS 202.

      - Grade 40 @ 300 pounds

    - Impact test – Large missile

      - Meets or exceeds FBC ASTM E1996 or Miami-Dade TAS 201.

      - Withstand 2x4 missile impact at all designated contact points without penetration.

    - Cyclic wind loading test

      - Meets or exceeds FBC ASTM E1886 or Miami-Dade TAS 203.

      - No failure or distress after subjected to the cyclic sequence to a design load (positive and negative.)

    - Upon request, the door manufacturer shall provide a test report from a qualified independent U.S. testing laboratory regularly engaged in testing doors to verify that products conform to test requirement as outlined.

- SUBMITTALS

  - Samples (as requested by architect):

    - Typical 6" long window profile with glazing beads.

    - Sample of specified muntin, showing welded intersections and glazing beads.

    - Hardware.

  - Shop drawings and manufacturer's literature:

    - Submit for approval shop drawings showing window and installation details, including anchorage, fastening and recommended sealing methods.

    - Dimensioned elevations showing window opening and window sizes.

    - The manufacturer shall not commence any work until shop drawings have been approved.

- PRODUCT, STORAGE AND HANDLING

  - The General Contractor shall be responsible for the protection and storage of the windows after delivery to the site.

  - Store in designated areas in an upright position on wood slats or on a dry floor in a manner that will prevent damage.  Ventilate canvas or plastic coverings to prevent humidity buildup.

- WARRANTY

  - Provide Hope’s standard 10 year Limited Material Warranty.

  - This guarantee is not intended to cover the bronze finish or any protective coating.

PART 2 - PRODUCT AND FABRICATION

- Manufacturers:

  - Furnish all labor and materials to complete the fabrication of windows as shown on architect's drawings and as specified herein.  All windows covered by this specification shall be domestically manufactured in the U.S.A.

  - Manufacturers:  Subject to compliance with requirements covered in this specification, provide products by Hope’s Windows, Inc. – Jamestown, NY (phone: 716-665-5124, e-mail: sales@hopeswindows.com) whose name and products are used to establish the standard of workmanship.

  - Substitutions:  Written approval necessary 10 days prior to bid through submission of the following:

    - Full-size window/door samples matching required scope.

    - Applicable test reports as outlined in Quality Assurance Section 1.2

    - List of (5) recently completed projects of similar size and scope.

- MATERIALS

  - Double weatherstripped windows shall be manufactured from solid extruded bronze profiles.

    - Profiles made from new bronze with flanges rolled integrally at the mill.

    - Perimeter frames and ventilator profiles shall have glazing rebates providing an unobstructed glazing surface of at least 3/4".

    - Glazing rebate surfaces must be perpendicular to the web or stem of the profile.  Applied glazing rebate extensions and rebate surfaces that are tapered will not be acceptable.

    - Combined weight of frame and ventilator profiles shall be a minimum of 4.40 pounds per lineal foot. Frame profile alone shall not weigh less than 1.95 pounds per lineal foot.

    - The frame and ventilator profiles shall have integral grooves located in the exterior and interior bedding contacts for the reception of double weatherstripping.

  - Muntins shall be manufactured from solid extruded bronze alloy 385 with a minimum thickness of .141 inches.

    - Glazing rebate surfaces must be perpendicular to the stem of this profile.  Rebate surfaces that are tapered will not be acceptable Glazing beads shall be extruded aluminum Alloy 6063-T5 with a minimum thickness of .062 inches.

    - 1-3/8″ tee shall weigh 1.664 pounds per lineal foot and the 7/8″ tee shall weigh 1.375 pounds per lineal foot (specify)

  - Glazing beads shall be extruded bronze.

  - Weatherstripping shall be extruded vinyl, EPDM closed cell sponge, flexible silicone or polyethylene clad urethane foam.

  - Operable Hardware (Select from the following):

    - Top Hung-Out Ventilators:

      - Fastener: Brass or Bronze cam fastener.

      - Ventilators shall be hung on aluminum-bronze pivot with stainless steel pin.

      - Friction and limit device: Stainless steel with sliding brass shoe and screw adjusted friction.

    - Project-In Ventilators:

      - Fastener: Brass or Bronze cam fastener; or bronze spring catches for ventilators beyond reach.

      - Ventilators shall be hung on heavy-duty stainless steel four bar hinges with brass friction shoes.

    - Project-out ventilators with simplex (non-roto) operators:

      - Fastener: Brass or bronze cam fastener.

      - Ventilators shall be hung on heavy-duty stainless steel four bar hinges with brass friction shoes.

    - Project-out ventilators with roto operators:

      - Fastener: Brass or Bronze cam fastener.

      - Ventilators shall be hung on heavy-duty stainless steel four bar hinges with brass friction shoes.

      - Dual arm roto operators: Solid brass or bronze housing with stainless steel arms and gears.

    - Side hung-in ventilators:

      - Fastener: Brass or Bronze cam fastener.

      - Pivots: Aluminum-bronze pivot with stainless steel pin.

      - Friction and limit device: Stainless steel with sliding brass shoe and screw adjusted friction.

    - Side hung-out ventilators with simplex (non-roto):

      - Fastener: Brass or Bronze cam fastener.

      - Pivots: Aluminum-bronze pivot with stainless steel pin.

      - Friction and limit device: Stainless steel with sliding brass shoe and screw adjusted friction.

      - Sill pull: Bronze sill pull (when required).

    - Side hung-out ventilators with roto operators:

      - Fastener: Brass or Bronze cam fastener.

      - Pivots: Aluminum-bronze pivot with stainless steel pin.

      - Roto Operators: Solid brass or bronze housing with stainless steel arms and gears.

  - All screws that are furnished by Hope’s, for hardware, trim, covers, anchoring, weatherbars, water dams, screens, etc. shall be non-ferrous brass or stainless steel.

  - Bronze Insect Screens (Optional):

    - Frames shall be roll-formed 1/32” bronze sheet.

    - Bronze screens shall be .011 diameter wire, woven to 14 x 18 mesh count.  Mesh is available in several types of material; see screen section for type of mesh (specify) and further screen specifications.

  - Finish (select from the following):

    - Factory Finished (specify finish)

    - Field finished by others

- FABRICATION

  - Fabricate bronze windows in accordance with approved shop drawings.

  - Corners of frame and ventilator shall be mitered or coped then welded.  Exposed and contact surfaces shall be finished smooth flush with the adjacent surfaces.  All interior and exterior rail bar and muntin joints shall be face welded and ground smooth.

  - Muntins shall be tenoned and welded to the perimeter frame.  Muntin intersections shall be slotted, cross notched and welded

  - Glazing:

    - All windows shall be designed for inside or outside glazing.

    - Provide replaceable continuous glazing beads to suit the glass as specified.

    - Glazing beads shall be cut and shop fitted to each glass lite prior to shipment.

    - Manufacturer to provide correct glazing wedge and tape in accordance with the tested assembly.

  - Weatherstrip:

    - Continuous weatherstripping shall be applied to the integral weatherstrip grooves in the interior and exterior contact surfaces of the frame and ventilator profiles. Weatherstripping that is surface applied or requires an additional retainer or requires screws for application shall not be acceptable.

  - Operable Hardware (Select from the following):

    - Top Hung-Out ventilators:

      - Provide two fasteners per ventilator where sash width exceeds 4'-8".

      - Ventilators shall be hung on aluminum-bronze pivot with stainless steel pin.

      - Friction device shall be applied at the jamb(s) of the ventilator.

    - Project-In ventilators:

      - Provide two fasteners per ventilator where sash width exceeds 4'-8".

      - Ventilators shall be hung on heavy-duty stainless steel four bar hinges, having friction maintained by a sliding brass shoe with screw adjustment.

    - Project-out ventilators with simplex (non-roto):

      - Provide two fasteners per ventilator where sash width exceeds 4'-8".

      - Ventilators shall be hung on heavy-duty stainless steel four bar hinges, having friction maintained by a sliding brass shoe with screw adjustment.

    - Project-out ventilators with roto operators:

      - Provide two fasteners per ventilator where sash width exceeds 4'-8".

      - Ventilators shall be hung on heavy-duty stainless steel four bar hinges, having friction maintained by a sliding brass shoe with screw adjustment.

      - Handles of dual arm roto operators are to be shipped loose for field installation.

    - Side hung-in ventilators:

      - Casement ventilators shall be hung on pivots.

      - Provide three pivots when vent height exceeds 5'-6".

      - Provide double grip fasteners for ventilators over 5'-6" in height.

      - Friction device shall be applied at the head of the ventilator.

    - Side hung-out ventilators with simplex (non-roto):

      - Casement ventilators shall be hung on pivots.

      - Provide three pivots when ventilator height exceeds 5'-6".

      - Provide double grip fasteners for ventilators over 5'-6" in height.

      - Friction device shall be applied at the head of the ventilator.

      - Sill pulls shall be furnished for ventilators if, when opened, the cam fastener is beyond reach.

    - Side hung-out ventilators with roto operators:

      - Casement ventilator shall be hung on pivots.

      - Provide three pivots when ventilator height exceeds 5'-6".

      - Provide double grip fasteners for ventilators over 5'-6" in height.

      - Handles of roto-operators are shipped loose for field installation.

  - Bronze Insect Screens (Optional):

    - Bronze screen frames shall be finished to match the windows.

    - Bronze screens shall be rewire able to allow for mesh replacement.

    - Bronze screen fastenings shall permit easy attachment and removal from the interior.  See screen section for further specifications.

PART 3 - EXECUTION

- INSPECTION

  - Window openings shall conform to details, dimensions and tolerances shown on the window manufacturers approved shop drawings.

  - Conditions which may adversely affect the window installation must be corrected before installation commences.

  - The wash down of the adjacent masonry or surrounding substrate must be completed before erection commences to prevent damage to the finish by the cleaning materials.

- INSTALLATION

  - Windows specified under this section shall be installed by experienced personnel.

  - Install windows in openings in strict accordance with approved shop drawings.

    - Set units plumb, level and true to line, without warp or rack of frames.

    - Anchor units securely to surrounding construction with approved fasteners.

    - The exterior joints between the windows, trim and mullions shall be properly sealed water-tight with an approved sealant and neatly pointed.

  - Attach ventilator hardware, as required, and adjust ventilators to operate smoothly free from twist and to be weather-tight when closed.

  - Repair any abraded areas of the factory finish.

- CLEANING

  - Window installer shall leave window surfaces clean after installation and ready to receive glass and glazing.  The window installer will not be responsible for final cleaning.