### PART 1 - GENERAL

- DESCRIPTION

  - Work included:

    - Hot-rolled steel triple weatherstripped impact approved doors in single or double, swing-out configurations.

    - All door anchors, mullions, covers and trim.

    - Factory applied Hope’s Power of 5 Finishing System.

  - Related work specified elsewhere:

    - Glass, glazing and glazing materials, Section 08810.

    - Perimeter caulking, Section 07915.

    - Miscellaneous structural items, Section 05100.

- QUALITY ASSURANCE

  - Manufacturer shall have not less than 10 years experience in the fabrication of heavy custom steel doors and be a member of The Steel Window Institute (SWI).

  - Installation of doors shall be done by experienced door installers.

  - Allowable tolerances: Size dimensions + 1/16 inch.

  - Source quality control:

    - All doors shall comply with the International Building Code (IBC), the International Residential Code (IRC), the Florida Building Code (FBC) and the Dade County Product Control Division.

    - Air infiltration test:

      - Products must be independently lab tested and meet or exceed ASTM E283 or Miami-Dade Testing Application Standard TAS 202.

      - Maximum air infiltration 0.30 CFM/SQ Ft. with differential pressure across window unit of 1.57/ 6.24 PSF.

    - Water penetration test:

      - Products must be independently lab tested and meet or exceed ASTM E331 or Miami-Dade TAS 202.

      - No water penetration for 15 minutes when window is subjected to a rate of flow of 5 gal./hr./sq. ft. with differential pressure across window unit of up to 8.25 PSF (dependent on 15% design pressure for areas requiring HVHZ).

    - Field test:

      - Field testing criteria (when applicable) shall be in accordance with AAMA 502-12

    - Structural test:

      - Meets or exceeds Miami-Dade TAS 202.  Door frames, muntins, mullions should be design to withstand ____ design pressure per required ASCE 7.

    - Forced entry test:

      - Meets or exceeds Miami-Dade TAS 202.

      - Grade 40 @ 300 pounds.

    - Impact test – Large missile:

      - Meets or exceeds Miami-Dade TAS 201.

      - Withstand 2x4 missile impact at all designated contact points without penetration.

    - Cyclic wind loading test:

      - Meets or exceeds FBC ASTM E1886 or Miami-Dade TAS 203.

      - No failure or distress after subjected to the cyclic sequence to a design load (positive and negative.)

    - Quality of Power of 5 non-coastal finishing process shall meet or exceed the following designations (non-coastal application – greater than (1) mile from salt water and/or similar corrosive  environments):

      - Paint Blistering:  ASTM D714.

      - Humidity:  ASTM D4585.

      - Salt Spray (Fog):  ASTM B117.

      - Painted Products in Corrosive Environments:  ASTM D1654.

      - Cyclic Fog/Dry (Prohesion):  ASTM G85.

      - Slat Fog/UV Painted Metal:  ASTM D5894.

      - Pull off Strength of Coating:  ASTM D4541.

    - Quality of Power of 5 coastal finishing process shall meet or exceed the following designations (Coastal application – less than (1) mile from salt water and/or similar corrosive environments):

      - Acid Pickling:  SSCP-SP8.

      - Hot Dip Galvanize:  ASTM A123.

      - Adhesion:  ASTM D3359, no loss.

      - Hardness:  ASTM D3363 (pencil), H min.

      - Salt Spray:  ASTM B117, passes 3000 hrs.

      - Humidity:  ASTM D2247, 3000 hours, few #8 blisters.

      - Impact Resistance (3mm):  ASTM D2794, no loss.

      - Color Retention:  ASTM D2244, 5 year less than or equal to 5 delta E.

      - Chalk Resistance:  ASTM D4214, #8 rating.

      - Gloss Retention:  ASTM D523, greater than or equal to 30 percent retention.

    - Upon request, the door manufacturer shall provide a test report from a qualified independent U.S. testing laboratory regularly engaged in testing doors to verify that products conform to test requirement as outlined.

- SUBMITTALS

  - Samples (as requested by architect)

  - Shop drawings and manufacturer's literature:

    - Submit for approval shop drawings showing door and installation details, including anchorage, fastening and recommended sealing methods.

    - Dimensioned elevations showing door opening and door sizes.

    - The manufacturer shall not commence any work until shop drawings have been returned “approved” with a stamp or signature.

    - Color charts for finishes.

- PRODUCT, STORAGE AND HANDLING

  - The General Contractor shall be responsible for the protection and storage of the doors after delivery to the site.

  - Store in designated areas in an upright position on wood slats or on a dry floor in a manner that will prevent damage.  Ventilate canvas or plastic coverings to prevent humidity buildup.

- WARRANTY

  - Provide Hope's standard 10 year Limited Warranty.

PART 2 - PRODUCT AND FABRICATION

- MANUFACTURERS:

  - Furnish all labor and materials to complete the fabrication of doors as shown on architect's drawings and as specified herein.  All doors covered by this specification shall be domestically manufactured in the U.S.A.

  - Manufacturers:  Subject to compliance with requirements covered in this specification, provide products by Hope’s Windows, Inc. – Jamestown, NY (phone: 716-665-5124, e-mail: sales@hopeswindows.com) whose name and products are used to establish the standard of workmanship.

  - Substitutions:  Written approval necessary 10 days prior to bid through submission of the following:

    - Full-size window/door samples matching required scope.

    - Applicable test reports as outlined in Quality Assurance Section 1.2

    - List of (5) recently completed projects of similar size and scope.

  - Basis of Design: Hope’s Landmark175 Impact Series.

- MATERIALS

  - Heavy custom triple weather stripped doors shall be manufactured from solid hot rolled steel profiles.

    - Profiles made from steel with flanges rolled integrally at the mill.

    - Perimeter frames and ventilator profiles shall have glazing rebates providing an unobstructed glazing surface of at least 5/8".

    - Glazing rebate surfaces must be perpendicular to the web or stem of the profile.  Applied glazing rebate extensions and rebate surfaces that are tapered will not be acceptable.

    - Combined weight of frame and ventilator profiles shall be a minimum of 3.50 pounds per lineal foot. Frame profile alone shall not weigh less than 1.80 pounds per lineal foot.

    - All steel profiles must be a minimum of 1-3/4″ in depth.

    - The frame and ventilator profiles shall have integral grooves located in the exterior and interior bedding contacts for the reception of triple weatherstripping.

    - Kickpanels shall be of steel construction.

    - Lockboxes shall be of steel construction.

  - Muntins (select from items 1 or 2 below):

    - True Divided Lite muntins:

      - Muntins shall be manufactured from solid hot-rolled steel, size to be determined by design.

      - Glazing rebate surfaces must be perpendicular to the stem of this profile.  Rebate surfaces that are tapered will not be acceptable.

      - 1-3/4” tee shall weigh min. 1.57 pounds per lineal foot.

      - All steel muntin profiles must be a minimum of 1-3/4″ in depth.

    - Simulated Divided Lite muntins  (select from items below):

      - Hot-rolled stainless steel muntin – #84H profile shall be solid hot-rolled from stainless steel with tapers rolled integral at the mill. #84H muntins shall be solidly welded to perimeter framing and dressed smooth. 10 gauge flat profile muntins shall be solidly welded to perimeter framing and dressed smooth. Opposite side muntin shall be extruded aluminum Alloy 6063-T5 matching the profiles as detailed.

      - Interior/exterior muntin - Profiles shall be extruded aluminum Alloy 6063-T5 matching the profiles as detailed.

  - Glazing beads shall be extruded aluminum alloy 6063-T5 matching the profiles as detailed.

  - Weatherstripping shall be extruded EPDM closed cell sponge, closed cell neoprene, flexible silicone, or polyethylene clad urethane foam.

  - All screws that are furnished by Hope’s, for hardware, trim, covers, anchoring, weatherbars, water dams, screens, etc. shall be non-ferrous brass or stainless steel.  Glazing bead screws are plated steel.

  - Power of 5 Finishing  (Select from item #1 for non-coastal applications or from item #2 for coastal applications):

    - Non-Coastal – greater than (1) mile from salt water and/or similar corrosive environments.

      - Cleaning.

      - Pretreatment.

      - Epoxy E-Coat primer.

      - Epoxy powder primer.

      - Ultrathane polyurethane top coat.

    - Coastal – within (1) mile of salt water and/or similar corrosive environments.

      - Cleaning.

      - Pretreatment with Hot-Dip Galvanizing.

      - Brush-Off Blast Cleaning.

      - Epoxy Powder Primer.

      - Polyester Powder Top Coat or Ultrathane polyurethane Top Coat.

- FABRICATION

  - Fabricate steel doors in accordance with approved shop drawings.

  - Corners of frame and ventilator shall be mitered or coped then solidly welded.  Exposed and contact surfaces shall be finished smooth flush with the adjacent surfaces.  All interior and exterior rail bar and muntin joints shall be face welded and ground smooth.

  - Lockboxes and Kickpanels

    - Lockboxes and Kickpanels shall be of steel profiles as shown on the shop drawings.

  - Muntins (select from 1 and 2):

    - True Divided Lite muntins shall be coped and welded to the perimeter frame.

    - Simulated Divided Lite Grids:

      - Hot-rolled exterior muntin - #84H profile shall be mitered or coped and fully back welded at all intersections.  10 gauge steel flats shall be welded flush to vents, size of flat to be determined by design.

      - Interior/exterior applied muntins - Profile shall be precut to meet perimeter frame. The intersections shall be milled to the extrusion profile. The muntin components shall be applied to the face of the glass with .045” VHB™ double adhesive tape after glazing.

  - Glazing:

    - All doors shall be designed for inside or outside glazing.

    - Provide replaceable continuous snap-in glazing beads to suit the glass as specified.

    - Glazing beads shall be cut and shop fitted to each glass lite prior to shipment.

    - Manufacturer to provide correct glazing wedge and tape in accordance with the tested assembly.

  - Weatherstrip:

    - All door leaves shall receive continuous triple weatherstripping that shall be applied to the integral weatherstrip grooves in the interior and exterior contact surfaces of the head and jambs of the frame and door profiles.  Weatherstripping that is surface applied or requires additional retainer or requires screws for application shall not be acceptable.  Sill weatherstripping shall be either surface applied to the face of the door, applied to an integral T-slot in the threshold, or attached to the underside of the door.

- FACTORY FINISHING (Select from items A or B below)

  - Power of 5 (non-coastal):

    - Cleaning:

      - All hot-rolled steel profiles must be acid pickled as defined by SSPC – SP8 to ensure a pristine, white metal substrate prior to fabrication.

    - Pretreatment:

      - Following welding and all machining operations, hot-rolled products and accessories are subjected to the following pretreatments geared specific to projects proximity to corrosive environment. Cold-rolled, formed sheet steel components are manufactured from A60 galvannealed sheet and subjected to applicable processes outlined below.

      - Inland locations (12-stage process) – Greater than one (1) mile from salt water and/or similar corrosive environments.

        - Alkaline cleaning spray.

        - Alkaline cleaning submersion.

        - Water immersion rinse combo.

        - Water immersion rinse clean.

        - Acid immersion.

        - Neutralizing rinse.

        - Water immersion rinse clean.

        - Conditioner immersion.

        - Zinc phosphate immersion.

        - Rinse immersion.

        - Sealer immersion.

        - Water reverse osmosis rinse immersion.

3.     Epoxy E-coat Primer:

All pickled and pretreated frames and accessories are immersed into an electrostatic (E-coat) bath of PPG epoxy primer to ensure all substrates are encapsulated evenly and completely. Use of spray primers only will not be an acceptable alternative to this process due to benefits from additional cleaning and frame submersion.

- Permeate spray.

- Permeate rinse.

- Epoxy primer immersion and electrostatic encapsulation

- Water reverse osmosis rinse

- Oven-cure, 45 minutes @ 350° F

4.   Epoxy Powder Primer:

Following pre-treatments and E-coat system, all frames and accessories shall receive an abrasion resistant powder coating prior to final top-coat.

    - Powder is applied electrostatically over cured E-coat to a dry film thickness (DFT) of 2.0 – 3.0 mils.

    - Parts oven baked at 325° F to completely cure prior to final top coat.

    - Powder coat is intended as an intermediate finish applied prior to the final finish top coat.

5.   Ultrathane Polyurethane Top Coat.

Following all pre-treatments, e-coat and powder abrasion layer, all products shall receive Hope’s ultrathane polyurethane finish with touch-up capability, low chalking and fading characteristics, unlimited color matching, and 70,000+ standard colors, including metallics.

6.   Power of 5 Overview (non-coastal):

    - Combined overall dry film thickness (DFT) shall be a minimum of 4.6 mils (inland locations)

    - Overall process shall provide full documented compliance with the following criteria:

      - Acid Pickling:  SSPC-SP8.

      - Paint Blistering:  ASTM D714.

      - Humidity:  ASTM D4585.

      - Painted Products in Corrosive Environment:   ASTM D1654.

      - Salt Spray (Fog):  ASTM B117.

      - Cyclic Fog/Dry (Prohesion):  ASTM G85.

      - Salt Fog/UV Painted Metal:  ASTM D5894.

      - Pull Off Strength of Coating:  ASTM D4541.

  - Power of 5 (coastal):

    - Cleaning:

All hot-rolled steel profiles must be acid pickled to white metal as defined by SSPC – SP8 creating a pristine, white metal substrate which is paramount to achieving ultimate finish performance.

    - Pretreatment:

Following welding and all machining operations, hot-rolled products and accessories are subjected to the following pretreatment process.

      - Hot-Dip Galvanize.

      - Parts are cleaned and immersed in a molten pool of pure zinc per ASTM A123 to create a dry film       thickness (DFT) of 4 – 8 mils of cathodic protection.

      - Quality Control.

      - Mechanical Cleaning.

    - Brush-Off Blast Cleaning:

      - Galvanized parts shall be brush-off blast cleaned in accordance with SSPS-SP16 to profile surface prior to primer application.

      - All Parts shall be oven heated prior to finishing to prevent outgassing through epoxy primer.

    - Epoxy Powder Primer:

Following pretreatment and brush-off blast cleaning all parts shall receive an abrasion resistant epoxy powder coat to ensure ultimate substrate protection. Epoxy powder primer is intended as an intermediate finish applied prior to the final finish top coat.

      - Within 12 hours of hot-dip galvanizing pretreatment all frames and accessories shall receive an epoxy powder primer for abrasion resistance.

      - After hot-dip galvanizing has been cleaned, brush-off blasted, and oven outgassed – epoxy powder primer is applied electrostatically to a dry film thickness (DFT) of 2.0 – 4.0 mils.

      - Parts are oven baked in preparation of top coat application.

    - Polyester Powder Top Coat or Ultrathane Polyurethane Top Coat:

The top coat defines the final stage to the overall aesthetics and performance of the finished product.

        - Polyester powder top coat shall be applied immediately over epoxy powder primer.

        - Powder top coat is electrostatically applied to a dry film thickness (DFT) of 2.0 – 4.0 mils.

        - Parts are oven baked to fully cure.

    - Power of 5 Overview (coastal):

      - Combined overall dry film thickness (DFT) of hot-dip galvanizing, epoxy primer, and polyester powder top coat or Ultrathane Polyurethane Top Coat shall be between 8.0 – 16.0 mils.

      - Overall process shall provide full documented compliance with the following criteria (as applicable):

        - Acid Pickling:  SSCP-SP8.

        - Hot Dip Galvanize:  ASTM A123.

        - Adhesion:  ASTM D3359, no loss.

        - Hardness:  ASTM D3363 (pencil), H min.

        - Salt Spray:  ASTMB 117, passes 3000 hrs.

        - Humidity:  ASTM D2247, 3000 hours, few #8 blisters.

        - Impact Resistance (3mm):  ASTM D2794, no loss.

        - Color Retention:  ASTM D2244, 5 year less than or equal to 5 delta E.

        - Chalk Resistance:  ASTM D4214, #8 rating.

        - Gloss Retention: ASTM D 523, greater than or equal to 30 percent retention.

- OPERABLE HARDWARE

  - Pivots:

    - Hope’s #237 Pivot

  - Door Trim Sets

    - Hope’s Collection One

      - Levers

      - Rosette Trim

      - Escutcheon Trim

      - Key Cylinder

      - Thumbturn

    - Hope’s Collection Two

      - Levers

      - Rosette Trim

      - Escutcheon Trim

      - Key Cylinder

      - Thumbturn

    - Water Street Brass Collections

    - Architect to Specify Other

  - Locksets:

    - Accurate:

      - #9100 mortise lock

  - Thresholds:

    - Threshold shall be as shown (select material from items a or b):

      - Aluminum (powder coated)

      - Bronze (toned US10B or Bead Blast mill finish).

  - Threshold Pan System:

    - Threshold pan, riser & support channel shall be formed as shown (select material from items a or b):

      - Aluminum (powder coated)

      - Bronze (toned US10B or Bead Blast mill finish).

  - Weatherstripping

    - Silicone/EPDM

  - Top & Bottom Bolts

    - Hope’s #20-50M3 surface mounted

  - Friction/Limit Device

    - Friction Adjuster

PART 3 - EXECUTION

- INSPECTION

  - Door openings shall conform to details, dimensions and tolerances shown on the door manufacturer's approved shop drawings.

  - Conditions which may adversely affect the door installation must be corrected before installation commences.

  - The wash down of the adjacent masonry or surrounding substrate must be completed before erection commences to prevent damage to the finish by the cleaning materials.

- INSTALLATION

  - Doors specified under this section shall be installed by experienced personnel.

  - Install doors in openings in strict accordance with approved shop drawings.

    - Set units plumb, level and true to line, without warp or rack of frames.

    - Anchor units securely to surrounding construction with approved fasteners.

    - The exterior joints between the doors, trim and mullions shall be properly sealed watertight with an approved sealant and neatly pointed.

  - Attach door hardware, as required, and adjust doors to operate smoothly free from twist and to be weathertight when closed.

  - Attach loose muntin grids per approved shop drawings, if applicable.

  - Repair any abraded areas of the factory finish.

- CLEANING

  - Door installer shall leave door surfaces clean after installation and ready to receive glass and glazing.  The door installer will not be responsible for final cleaning.